Aquatic Venue Log Sheet – Chlorine / Bromine	Venue Name (Leisure Pool, Spa, Dive Well, etc.):________________________________

| Date | Time | Cl \ Br Daily | Total Cl Daily | ORP Daily | pH Daily | Temp Daily | Total Alkalinity Weekly | CYA Weekly | Calcium Hardness Weekly | Salt Weekly | Saturation Index Monthly | UV / Ozone Daily | Chemicals Added/Other Adjustments; Chemicals to Order and/or Maintenance Items Addressed | Initials |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Cl=Chlorine     ORP=Oxidation Reduction Potential     CYA=Cyanuric Acid     UV=Ultraviolet

Water Quality Parameters

| AV Type | Chlorine without CYA | Chlorine with CYA | Bromine | pH | Reuse Ratio |
| --- | --- | --- | --- | --- | --- |
| Lap Pool, Leisure Pool, Dive Pool, etc. | 1 – 5 ppm Close <1ppm or >10ppm | 2 – 5 ppm Close <2ppm or >10ppm | 3 – 8 ppm Close <3ppm or >8ppm | 7.0 – 7.8 Close <6.5 >8.0 |  |
| Spa | 3 – 5 ppm Close <3 ppm or >10 ppm | 3 – 5 ppm Close <3ppm or >10ppm | 4 – 8 ppm Close <3ppm or >8ppm | 7.0 – 7.8 Close <6.5 >8.0 |  |
| Interactive Water Play: Splash Pad, Lazy River, Exercise, etc. | 1 – 5 ppm Close <1ppm or >10ppm | 2 – 5 ppm Close <2ppm or >10ppm | 3 – 8 ppm Close <3ppm or >8ppm | 7.0 – 7.8 Close <6.5 >8.0 | Splash Pad 3:1 |

AV=Aquatic Venue     CYA=Cyanuric Acid

|  | Combined Cl | Total Alkalinity | CYA | Calcium Hardness | Salt | Saturation Index | UV | Ozone |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All Pools | ≤ 0.4 ppm | 60-180 ppm | ≤ 90 ppm & ≤ 45:1 Ratio | ≤ 2500 ppm | ≥ 3500 ppm or manufacturer recommendations | -0.5 to +0.5 | Manufacturer recommendations | ≤ 0.1 ppm for Float Tanks, all other pools, manufacturer recommendations |

Turnover Times

| Type of Pool | Maximum Turnover Time |
| --- | --- |
| Activity Pool | 2 hours or less |
| Diving Pool | 8 hours or less |
| Interactive Water Play* | 0.5 hour or less |
| Lazy River | 2 hours or less |
| Plunge Pool | 1 hour or less |
| Spa, ≥93℉ - 104℉ | 0.5 hour or less |
| Slide Runout | 1 hour or less |
| Therapy Pool* and Exercise Pool ≤ 72⁰–93⁰F, >2500 gal/person | 4 hours or less |
| Therapy Pool* and Exercise Pool ≤ 72⁰–93⁰F, >450 gal/person | 2 hours or less |
| Therapy Pool* and Exercise Pool ≤ 72⁰–93⁰F, ≤450 gal/person | 1 hour or less |
| Wading Pool* | 1 hour or less |
| Wave Pool | 2 hours or less |
| All Other Pools | 6 hours or less |

*=Shall Have Secondary Treatment (UV, Ozone, AOP)